Theory of Flight

Homework 2

Name:_______________________________________  Date:_______________

1.	The corrections that must be made to Indicated Airspeed (lAS) to obtain Calibrated Airspeed (CAS) are:

a.	position error and compressibility.

b.	instrument error and position error.

c.	instrument error and density error.

d.	position error and density error.

2.	The correction that must be made to CAS to obtain Equivalent Airspeed (EAS) is called

3.	The correction from EAS to True airspeed(TAS) is dependent upon

a.	temperature ratio alone.

b.	density ratio alone.

c.	pressure ratio alone.

d.	none of the above.

4.	Briefly what does the airspeed indicator measure?

5.	Density ratio σ (sigma) is:

a.	measured in slugs per cubic foot.

b.	equal to pressure ratio divided by temperature ratio.

c.	equal to ambient density divided by sea level standard pressure.

d.	none of the above.

6.	An airplane is operating from an airfield which has a barometric pressure of 26.81 in. Hg and a runway temperature of 100˚ F. Calculate (or find from the ICAO tables) the following:

A)	pressure ratio ________

B)	pressure altitude _______

C)	Temperature ratio _______

D)	Density ratio _______

E)	Density altitude _______

7.	Fill in the values	density ratio σ = 0.968

1

2

3

At station 1:		At station 2:				At station 3:

A = 12ft2	A = 6ft2					A = _______

V = 50kts	V = _______				V = 25kts

P = 2000psf	P = _______				P = _______

q = ______	q = _______				q = _______

H = ______	H = _______				H = _______

8. 	The density ratio for various altitudes are listed below. Assuming
an indicated airspeed of 100 knots, find the corresponding TAS for each altitude:

Altitude	σ		IAS		TAS

Sea Level 	1.0		100		____

5000	0.8617		100		____
	10,000	0.7385		100		____
	20,000	0.5328		100		____
	30,000 	0.3741		100		____

40,000	0.2462		100		____

9.	At a TAS of 90 Knots what is the dynamic pressure of q at sea

level?___________  What is q if the density ratio σ = 0.5?_______________

10.	If the density ratio σ = .7, what is the density in slugs per cubic foot?

11.	A pitot-Static system senses a dynamic pressure of 135.6 lbs/ ft2.

What is the IAS at Sea Level?__________

What is the TAS as Sea Level?_________

What is the IAS at 40,000 ft? __________

What is the TAS at 40,000? ___________

12.	Compute the value of q (dynamic pressure) for an airplane flying at 150 knots IAS. Make this computation for the following altitudes and density ratios:

Altitude		σ		q

Sea Level	1.0		_______

5,000 ft	0.8617		_______

10,000 ft	0.7385		_______

20,000 ft	0.5328		_______